Ford PUMA ST Specifications

Performance and Economy

| Puma ST |  |  |  | Performance |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Engine | Power (PS) | CO2 (g/km) 2 | Fuel consumption l/100 km 2 |  | Max speed (km/h) | 0-100 km/h (sec) | 50-100 km/h (sec) 3 |
| 1.0-litre EcoBoost Hybrid (7-spd Powershift automatic) | 170 1 | 144 | 6.3 |  | 210 | 7.4 | N/A |
| 1.5-litre EcoBoost (6-spd manual) | 200 | 149-155 | 6.6-6.8 |  | 220 | 6.7 | 5.9 |

1 Maximum performance determined by test procedure UN Global Technical Regulation-21 (Jan 2021).

2 CO2 emission and fuel efficiency ranges may vary according to vehicle variants offered by individual markets.

The declared WLTP fuel/energy consumptions, CO2-emissions and electric range are determined according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EU) 2017/1151 as last amended. The applied standard test procedures enable comparison between different vehicle types and different manufacturers.

3 In 4th gear.

WEIGHTS AND DIMENSIONS

|  | Kerb weight (kg) 4 | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) 5 | Max. Towable Mass (unbraked) (kg) 5 | Nose weight (kg) | Roof load (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0-litre EcoBoost Hybrid 170 PS 7-spd Powershift automatic | 1394 | 1845 | 2745 | 900 | 665 | 75 | 25 |
| 1.5-litre EcoBoost 200 PS 6-spd manual | 1358 | 1815 | 2565 | 750 | 640 | 75 | 25 |

4 Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted. Weights represent base model specification without panoramic glass roof.

5 Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight.

Dimensions

| Dimensions (mm unless stated) | Puma ST 1.0-litre EcoBoost Hybrid | Puma ST 1.5-litre EcoBoost |
| --- | --- | --- |
| Exterior |  |  |
| Overall length without tow bar | 4226 | 4226 |
| Overall width mirrors extended/folded | 1930/1805 | 1930/1805 |
| Overall height (with shark fin antenna, unladen) | 1533 | 1533 |
| Wheelbase | 2588 | 2588 |
| Front track | 1565 | 1565 |
| Rear track | 1518 | 1518 |
| Front overhang | 875 | 875 |
| Rear overhang | 763 | 763 |
| Min ground clearance (GVM) | 131 | 131 |
| Min ground clearance (kerb) | 152 | 152 |
| Approach Angle (degrees) | 13.8 | 13.8 |
| Departure Angle (degrees) | 24.0 | 24.0 |
| Interior |  |  |
| Front headroom (without panorama roof) | 1000 | 1000 |
| Front headroom (with panorama roof) | 983 | 983 |
| Front legroom (maximum with seat in rear-most mid-height position) | 1127 | 1127 |
| Front shoulder room | 1348 | 1348 |
| Front hip room | 1311 | 1311 |
| Rear headroom (without panorama roof) | 965 | 965 |
| Rear legroom (maximum with seat in rear-most mid-height position) | 877 | 877 |
| Rear shoulder room | 1320 | 1320 |
| Rear hip room | 1278 | 1278 |
| Luggage capacity (litres) 6 |  |  |
| 5-seat mode, laden to package tray (with tyre repair kit) | 456 | 456 |
| 2-seat mode, laden to roof (with tyre repair kit) | 1216 | 1216 |
| Luggage compartment dimensions |  |  |
| Maximum loading height | 865 | 865 |
| Load length at floor to 1st row (tyre repair kit) | 1472 | 1472 |
| Load length at floor to 2nd row seats | 725 | 725 |
| Load width between wheelhouses | 1000 | 1000 |
| Load opening width (floor) | 982 | 982 |
| Lift-over height (kerb) | 754 | 754 |
| Fuel tank capacity (litres) |  |  |
| Petrol | 42 | 45 |

6 Measured in accordance with ISO 3832. Dimensions may vary dependent on the model and equipment fitted.

ADVANCED DRIVER-ASSISTANCE SYSTEMS 7, 8

Adjustable Speed Limiter

Auto high beam

Auto lighting and rain-sensing wipers

Blind Spot Information System

Cross Traffic Alert with Active Braking

Cruise Control

Driver Alert

Enhanced Active Park Assist

Forward Collision Warning

Front and rear parking sensors

Hill Launch Assist

Intelligent Speed Limiter

Local Hazard Information

Lane Keeping Alert

Post-Collision Braking

Pre-Collision Assist with Active Braking

Rear wide-view camera

Selectable Drive Modes

Traffic Sign Recognition

Wrong Way Alert

COMFORT AND CONVENIENCE 8

B&O Sound System

Ford MegaBox

FordPass Connect modem

Hands-free tailgate

Adjustable Lumbar support seats

Openable panoramic roof

SYNC 3 with 8-inch touchscreen

Wireless charging pad

7 Driver-assist features are supplemental and do not replace the driver’s attention, judgment and need to control the vehicle. It does not replace safe driving. See Owner’s Manual for details and limitations.

8 Feature availability dependent on vehicle specification.

STEERING

| System | Rack and pinion with Electronic Power Assisted Steering (EPAS) |
| --- | --- |
| Ratio | 11.4:1 |
| Turning circle (m) kerb-to-kerb | 11.2 |

CHASSIS

| Front suspension | Independent suspension with MacPherson struts, Hitachi twin-tube frequency reactive shock absorbers and 24.2 mm x 6 mm diameter anti-roll bar |
| --- | --- |
| Rear suspension | Twist-beam rear suspension with toe-correcting angled bushes and 28 mm anti-roll bar. In-car stiffness of 2000 Nm/deg. Force vectoring springs located below floor and separate Hitachi twin-tube frequency reactive shock absorbers |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs. Solid rear discs. Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD), Electronic Stability System (ESP) and Emergency Brake Assist (EBA). Optional autonomous emergency braking (AEB) as part of Pre-Collision Assist with Active Braking and Cross Traffic Alert with Active Braking |  |
| Disc/Drum dimensions (mm) | Ø325 x 27 | Ø271 x 11 |
| Piston dimensions (mm) | Ø57 | Ø36 |

WHEELS & TYRES

8J x 19-inch alloy wheels with Magnetite finish and Continental Sport Contact 7 225/40 ZR19 XL 93Y

HYBRID ENGINES

|  |  | 1.0-litre EcoBoost Hybrid (170 PS) |
| --- | --- | --- |
| Type |  | In-line three-cylinder turbo petrol, 48-volt mild hybrid, Ti-VCT, transverse |
| Displacement | cm3 | 999 |
| Bore | mm | 71.9 |
| Stroke | mm | 82.0 |
| Compression ratio |  | 10.0:1 |
| Max power 1 | PS (kW) | 170 (125) |
|  | at rpm | 5750 |
| Max torque steady state (ICE) | Nm | 200 |
|  | at rpm | 2000-5500 |
| Max torque performance 1 | Nm | 248 |
|  | at rpm | 3000 |
| Valve gear Cylinders |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |
|  |  | 3 in-line, cylinder deactivation technology |
| Cylinder head |  | Cast aluminium |
| Cylinder block |  | Cast iron |
| Camshaft drive |  | Chain drive with hydraulic tensioner |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. FGEC software |
| Fuel injection |  | 250 bar (max) high pressure direct fuel injection with 5-hole injectors |
| Emission level |  | Euro 6d |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |
| Turbocharger |  | Fixed geometry turbocharger |
| Electric motor |  | 11.5kW Belt Integrated Starter-Generator |
| Battery |  | 48V 8Ah Li-ion air cooled battery pack |
| Lubrication system |  | Electronically controlled variable displacement oil pump for improved fuel economy |
| Cooling system |  | Split cooling system with 2 thermostats. Cam driven water pump for reduced power consumption |
| Transmission |  | 7-speed Powershift dual-clutch automatic |
| Gear ratios |  | 7th 0.638 6th 0.771 5th 0.851 4th 1.114 3rd 1.596 2nd 2.824 1st 4.462 R 2.824 FDR 4.176 (ratios 1,2,6,7) 4.733 (ratios 3,4,5,R) |

PETROL ENGINEs

|  |  | 1.5-litre EcoBoost (200 PS) |
| --- | --- | --- |
| Type |  | In-line three-cylinder turbo petrol, Ti-VCT, transverse |
| Displacement | cm3 | 1497 |
| Bore | mm | 84.0 |
| Stroke | mm | 90.0 |
| Compression ratio |  | 9.7:1 |
| Max power | PS (kW) | 200 (147) |
|  | at rpm | 6000 |
| Max torque steady state | Nm | 320 |
|  | at rpm | 2500-3500 |
| Valve gear Cylinders |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |
|  |  | 3 in-line, cylinder deactivation technology |
| Cylinder head |  | Cast aluminium |
| Cylinder block |  | Cast aluminium |
| Camshaft drive |  | Chain drive with hydraulic tensioner |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |
| Engine management |  | Bosch MG1CS016 with CAN-Bus and individual cylinder knock control. FGEC software |
| Fuel injection |  | High pressure direct fuel injection with 6-hole injectors (DI) plus 3 low-pressure individual port injectors (PFI) |
| Emission level |  | Euro 6d |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |
| Turbocharger |  | Continental RAAX low-inertia turbocharger |
| Lubrication system |  | Two-stage variable displacement oil pump |
| Cooling system |  | Single thermostat |
| Transmission |  | 6-speed manual with optional Quaife Limited-Slip Differential |
| Gear ratios |  | 6th 0.651 5th 0.775 4th 0.971 3rd 1.290 2nd 1.952 1st 3.583 R 3.333 FDR 4.563 |

# # #

Note: The data information in this press release was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

About Ford Motor Company

Ford Motor Company (NYSE: F) is a global company based in Dearborn, Michigan, committed to helping build a better world, where every person is free to move and pursue their dreams.  The company’s Ford+ plan for growth and value creation combines existing strengths, new capabilities and always-on relationships with customers to enrich experiences for customers and deepen their loyalty.  Ford develops and delivers innovative, must-have Ford trucks, sport utility vehicles, commercial vans and cars and Lincoln luxury vehicles, along with connected services.  The company does that through three customer-centered business segments:  Ford Blue, engineering iconic gas-powered and hybrid vehicles; Ford Model e, inventing breakthrough EVs along with embedded software that defines always-on digital experiences for all customers; and Ford Pro, helping commercial customers transform and expand their businesses with vehicles and services tailored to their needs.  Additionally, Ford is pursuing mobility solutions through Ford Next, and provides financial services through Ford Motor Credit Company.  Ford employs about 173,000 people worldwide.  More information about the company and its products and services is available at corporate.ford.com.

Ford, a global American brand woven into the fabric of Europe for more than 100 years, is committed to freedom of movement that goes hand-in-hand with looking after the planet and each other. The company’s Ford+ plan, with Model e, Ford Pro and the Ford Blue business units is accelerating its European transformation to an all-electric and carbon neutral future by 2035. The company is driving forward with bold, new EVs, each one designed with European drivers in mind and innovating with services to help people connect, communities grow, and businesses thrive. Selling and servicing Ford vehicles in 50 individual European markets, operations also include the Ford Motor Credit Company, Ford Customer Service Division and 14 manufacturing facilities (eight wholly owned and six unconsolidated joint venture facilities) with four centres based in Cologne, Germany; Valencia, Spain and at our joint venture in Craiova, Romania and Kocaeli, Türkiye.  Ford employs approximately 34,000 people at its wholly owned facilities and consolidated joint ventures and approximately 54,000 people including unconsolidated businesses across Europe. More information about the company, its products and Ford Credit is available at corporate.ford.com.