Technical specifications.

BMW iX3.

iX3 50 xDrive.

|  |  |  |  |  |  | BMW iX3 50 xDrive |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Category |  |  |  |  |  |  |
| Drive type / body style |  |  |  | Battery-electric vehicle (BEV) / Sports Activity Vehicle (SAV) |  |  |
| Body |  |  |  |  |  |  |
| No. of doors / seats |  |  |  |  |  | 5 / 5 |
| Length/width/height (unladen) | mm |  |  |  |  | 4782 / 1895 / 1635 |
| Wheelbase | mm |  |  |  |  | 2897 |
| Track, front/rear | mm |  |  |  |  | 1628 / 1633 |
| Turning circle |  | m |  |  |  | 12.1 |
| Weight, unladen (DIN/EU) | kg |  |  |  |  | 2285 / 2360 |
| Max. load to DIN | kg |  |  |  |  | 540 |
| Max. permissible weight | kg |  |  |  |  | 2825 |
| Max. axle load, front/rear | kg |  |  |  |  | 1305 / 1565 |
| Max. trailer load, braked (12%)/unbraked | kg |  |  |  |  | 2000 / 750 |
| Max. roofload/towbar download | kg |  |  |  |  | 75 / 80 |
| Luggage comp. capacity | l |  |  |  |  | 520 – 1750 |
| Additional storage comp. under bonnet |  |  |  |  | l | 58 |
| Air resistance | cX x A |  |  |  |  | 0.24 x 2.62 |
| Power Unit |  |  |  |  |  |  |
| Drive concept |  |  |  |  |  | Electric all-wheel drive, coordinated transmission of the drive torque from two electric motors to the front and rear wheels respectively, as required |
| Max. system output | kW/hp |  |  |  |  | 345 / 469 |
| Max. system torque | Nm |  |  |  |  | 645 |
| System power-to-weight ratio | kg/kW |  |  |  |  | 6.6 |
| Type of transmission |  |  |  |  |  | Automatic transmission, single-speed with fixed ratio |
| Electric motors |  |  |  |  |  |  |
| Motor technology |  |  |  |  |  | Sixth-generation BMW eDrive technology: electrically excited synchronous motor at the rear axle, asynchronous motor at the front axle, each sharing a housing with the power electronics and transmission; generator function for recuperating energy |
| Front Electric Motor |  |  |  |  |  |  |
| Motor designation |  |  |  |  |  | HF1001N0 |
| Peak output to ECE R 85 | kW/hp |  |  |  |  | 123 / 167 |
| at | rpm |  |  |  |  | 4607 |
| Max. torque | Nm |  |  |  |  | 255 |
| at | rpm |  |  |  |  | 0 – 4607 |
| Gear ratio | :1 |  |  |  |  | 8.801 |
| Rear Electric Motor |  |  |  |  |  |  |
| Motor designation |  |  |  |  |  | HD1002N0 |
| Peak output to ECE R 85 | kW/hp |  |  |  |  | 240 / 326 |
| at | rpm |  |  |  |  | 5000 |
| Max. torque | Nm |  |  |  |  | 435 |
| at | rpm |  |  |  |  | 0 – 5000 |
| Gear ratio | :1 |  |  |  |  | 9.606 |
| High-voltage Battery |  |  |  |  |  |  |
| Storage technology |  |  |  |  |  | Lithium-ion |
| Installation |  |  |  |  |  | Underfloor |
| Voltage | V |  |  |  |  | 698.9 |
| Energy capacity, net | kWh |  |  |  |  | 108.7 |
| Charging time, 0 – 100 % charge |  |  |  |  |  | 11h at 11 kW (16 A / 230 V, three-phase AC, wallbox) |
| Charging time, 0 – 100 % charge |  |  |  |  |  | 5h 45m at 22 kW (32 A / 230 V, three-phase AC, wallbox) |
| Charging time, 10 – 80 % charge |  |  |  |  |  | 21 min at 400 kW (800 A, DC, fast-charging station) |
| Additional range after 10 minutes of DC charging (max. charging rate) 1) |  |  | km |  |  | 309 – 372 |
| Charging |  |  |  |  |  |  |
| Max. charging rate AC, three-phase | kW |  |  |  |  | 11.0 (optional: 22.0) |
| Max. charging rate DC | kW |  |  |  |  | 400 |

|  |  |  |  |  |  |  |  |  | BMW iX3 50 xDrive |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bidirectional Charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vehicle-to-Load, max. rate (AC) |  |  |  |  |  |  | kW |  |  |  | optional: 3.7 |  |  |  |
| Vehicle-to-Home, max. rate (DC) |  |  |  |  |  |  | kW |  |  |  | 11.0 |  |  |  |
| Vehicle-to-Grid, max. rate (DC) |  |  |  |  |  |  | kW |  |  |  | 11.0 |  |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suspension, front |  |  |  |  |  |  |  |  |  |  | Double-joint spring strut axle in lightweight aluminium-steel construction with torque arm and anti-roll bar. Trapezoidal shear panels forming connection between axle subframe, high-voltage battery and body |  |  |  |
| Suspension, rear |  |  |  |  |  |  |  |  |  |  | Five-link axle in lightweight aluminium-steel construction with separately arranged springs and dampers |  |  |  |
| Brakes, front |  |  |  |  |  |  |  |  |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |
| Brake disc size |  |  |  | mm |  |  |  |  |  |  | 330 |  |  |  |
| Brakes, rear |  |  |  |  |  |  |  |  |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |
| Brake disc size |  |  |  | mm |  |  |  |  |  |  | 345 |  |  |  |
| Driving stability systems |  |  |  |  |  |  |  |  |  |  | Standard: Heart of Joy integrated drivetrain and driving dynamics management, integrated braking system, DSC incl. ABS, ASC and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, HDC (Hill Descent Control), trailer stability control |  |  |  |
| Safety equipment |  |  |  |  |  |  |  |  |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, interaction airbag between driver and front passenger, head airbags for front and rear seats, three-point inertia-reel seatbelts on all seats with belt tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |  |  |
| Steering |  |  |  |  |  |  |  |  |  |  | Electric Power Steering (EPS) with Servotronic function |  |  |  |
| Steering wheel turns (lock to lock) |  |  |  |  |  |  |  |  |  |  | 2.8 |  |  |  |
| Tyres, front/rear |  |  |  |  |  |  |  |  |  |  | 255/45 R20 105W XL |  |  |  |
| Rims, front/rear |  |  |  |  |  |  |  |  |  |  | 8.5J x 20 light-alloy |  |  |  |
| Performance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acceleration 0–100 km/h |  |  | s |  |  |  |  |  |  |  | 4.9 |  |  |  |
| Top speed |  |  | km/h |  |  |  |  |  |  |  | 210 (electronically limited) |  |  |  |
| Off-road Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Angle of approach/departure |  | ° |  |  |  |  |  |  |  |  | 18.6 / 20.2 |  |  |  |
| Breakover angle |  | ° |  |  |  |  |  |  |  |  | 15.9 |  |  |  |
| Ground clearance (unladen) |  |  |  |  | mm |  |  |  |  |  | 176 |  |  |  |
| Fording depth (at 5 km/h) |  | mm |  |  |  |  |  |  |  |  | 400 |  |  |  |
| Consumption / Range (WLTP combined) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electric power consumption as per EnVKV |  |  |  |  |  | kWh/100 km |  |  |  |  | 17.9 – 15.1 |  |  |  |
| Attainable electric power cons. in individual configuration |  |  |  |  |  | kWh/100 km |  |  |  |  | 17.9 – 15.1 |  |  |  |
| Electric power consumption motorway 2) |  |  |  |  |  | kWh/100 km |  |  |  |  | 22.7 – 19.4 |  |  |  |
| Range as per EnVKV |  |  |  |  |  | km |  |  |  |  | 679 – 805 |  |  |  |
| Attainable range in individual configuration |  |  |  |  |  | km |  |  |  |  | 679 – 805 |  |  |  |
| Environmental Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pass-by noise |  |  |  |  |  |  | dB(A) |  |  |  | 65 |  |  |  |
| Emission rating |  |  |  |  |  |  |  |  |  |  | Electric vehicle |  |  |  |
| CO2 class(es) as per EnVKV |  |  |  |  |  |  |  |  |  |  | A |  |  |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany

All figures are provisional as at the start of production in November 2025.

All of the stated equipment features, technical data and fuel/electric power consumption and emissions figures relate to the offering in the German market. The figures refer to a vehicle with basic configuration in Germany. These may vary depending on the wheel/tyre size and items of optional equipment selected.

Official electric power consumption and range figures were determined based on the prescribed WLTP measurement procedure and are in accordance with the Pkw-EnVKV. Further information can be found at www.bmw.de

1) The extra range added after ten minutes of high-power charging was calculated in accordance with ISO 12906 based on the WLTP cycle. Both this and charging performance are dependent on vehicle equipment, the battery’s charge level and age, battery temperature, the individual driving profile, use of auxiliary consumers, the ambient temperature and the charging station’s available charging power.

2) Corresponds to WLTP Extra High

Exterior and interior dimensions.

BMW iX3.

iX3 50 xDrive.

The dimensions indicated in the technical drawing are in millimetres and may vary depending on the model and items of optional equipment fitted.

BMW iX3.

iX3 50 xDrive with M Sport package.

The dimensions indicated in the technical drawing are in millimetres and may vary depending on the model and items of optional equipment fitted.