Ford MUSTANG BULLITT – EUROPEAN Specifications

PERFORMANCE AND ECONOMY

|  |  |  | Fuel consumption l/100 km (mpg)* |  |  | Performance |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Engine | Power (PS) | CO2 (g/km)* | Urban | Extra Urban | Combined | Max speed km/h (mph) | 0-100 km/h 0-62 mph (sec) |
| 5.0-litre V8 fastback (6-speed manual) | 460 | 277 | 16.6 (17.0) | 8.8 (32.1) | 12.4 (22.8) | 263 (163) | 4.6 |

*The declared Fuel/Energy consumptions, CO2 emissions and electric range are measured according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EC) 692/2008 as last amended. Fuel consumption and CO2 emissions are specified for a vehicle variant and not for a single car. The applied standard test procedure enables comparison between different vehicle types and different manufacturers. In addition to the fuel-efficiency of a car, driving behaviour as well as other non-technical factors play a role in determining a car's fuel/energy consumption, CO2 emissions and electric range. CO2 is the main greenhouse gas responsible for global warming.

From 1 September 2017, certain new vehicles will be type-approved using the World Harmonised Light Vehicle Test Procedure (WLTP) according (EU) 2017/1151 as last amended, which is a new, more realistic test procedure for measuring fuel consumption and CO2 emissions. From 1 September 2018 the WLTP will fully replace the New European Drive Cycle (NEDC), which is the current test procedure. During NEDC Phase-out, WLTP fuel consumption and CO2 emissions are being correlated back to NEDC. There will be some variance to the previous fuel economy and emissions as some elements of the tests have altered i.e., the same car might have different fuel consumption and CO2 emissions.

BODY

| Construction | Unitised welded steel body, aluminium bonnet and front wings |
| --- | --- |
| Body Styles | Fastback; Left-hand and Right-hand Drive |
| Final assembly location | Flat Rock, Michigan, USA |

ENGINE

| Configuration | Aluminium block and heads |
| --- | --- |
| Intake manifold | Composite shell-welded with runner pack and charge motion control valves |
| Exhaust manifold | Stainless steel tubular manifold |
| Valvetrain | DOHC, four valves per cylinder, twin independent variable camshaft timing |
| Valve diameter/lift (mm) | Intake 37.7 /13 Exhaust 32.0 /13 |
| Pistons | Cast aluminium |
| Connecting rods | Forged steel |
| Ignition | Distributor-less with coil-on-plug |
| Bore x stroke (mm) | 93.0 x 92.7 |
| Displacement (cc) | 5038 |
| Compression ratio | 12.0:1 |
| Engine control system | PCM |
| Power | 460 PS (338 kW) @ 7250 rpm |
| Torque | 529 Nm @ 4600 rpm |
| Fuel capacity (litre) | 61 |
| Fuel delivery | Dual-injection system featuring low-pressure port fuel injection and high-pressure direct injection |
| Oil capacity (litre) | 9.5 5W-20 with filter |

DRIVETRAIN

| Layout | Rear-wheel drive with limited-slip differential |
| --- | --- |

TRANSMISSION

|  | 5.0-litre V8 |
| --- | --- |
| Standard | 6-speed manual |
| First | 3.24 |
| Second | 2.10 |
| Third | 1.42 |
| Fourth | 1.00 |
| Fifth | 0.81 |
| Sixth | 0.62 |
| Final drive | 3.55 |

SUSPENSION

| Front | Double-ball-joint independent MacPherson strut and tubular stabiliser bar |
| --- | --- |
| Rear | Integral-link independent with coil springs, solid stabiliser bar and mono-tube dampers |
| Optional | MagneRide® Damping System with Monotube rear dampers and inverted monotube front dampers; suspension position sensors and Vehicle Dynamics Module |

BRAKES

| Type | Four-wheel power disc brakes with four-sensor, four-channel anti-lock braking system and AdvanceTrac electronic stability control |
| --- | --- |
| Front | 380 mm x 34 mm vented discs, Brembo six-piston 36 mm fixed aluminium callipers |
| Rear | 330 mm x 25 mm vented discs, Brembo single-piston 45 mm floating iron callipers, integral parking brake |

STEERING

| Type | Selectable electric power-assisted steering with rack-and-pinion gear |
| --- | --- |
| Ratio | 16:1 |
| Turning circle, kerb-to-kerb | 12.2 m |

WHEELS AND TYRES

| 5.0-litre V8 | Front: 255/40 R19 Tyres, 19 inch x 9 inch aluminium wheels | Rear: 275/40 R19 Tyres, 19 inch x 9.5 inch aluminium wheels |
| --- | --- | --- |

DIMENSIONS

| EXTERIOR |  |
| --- | --- |
| Wheelbase (mm) | 2720 |
| Overall length (mm) | 4789 |
| Overall width excluding mirrors (mm) | 1916 |
| Overall width including mirrors (RHD) (mm) | 2081 (2097) |
| Overall width mirrors folded (RHD) (mm) | 1957 (1985) |
| Overall height (max) (mm) | 1382 |
| Track, front/rear (mm) | 1582/ 1655 |
| Minimum ground clearance (kerb) (mm) | 144 |
| INTERIOR |  |
| Seating capacity | Four |
| Front headroom (mm) | 955 |
| Front legroom – maximum (mm) | 1146 |
| Front shoulder room (mm) | 1430 |
| Front hip room (mm) | 1395 |
| Rear headroom (mm) | 884 |
| Rear legroom (mm) | 737 |
| Rear shoulder room (mm) | 1327 |
| Rear hip room (mm) | 1203 |
| Loading Width Between Wheelhouses (mm) | 1010 |
| Load Opening Width at floor (mm) | 640 |
| Liftover Height (kerb) (mm) | 761 |
| Cargo volume (litre) ‡ | 408 |

‡Measured in accordance with ISO 3832. Information provided for base vehicle, except where specified. Dimensions may vary dependent on the model and equipment fitted.

WEIGHTS

|  | Kerb weight (kg)# | Gross Vehicle Mass (kg) |
| --- | --- | --- |
| 5.0-litre V8 fastback (6-speed manual) | 1818 | 2105 |

# Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted.

# # #

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

MagneRide is a trademark of BWI Group

About Ford Motor Company

Ford Motor Company is a global company based in Dearborn, Michigan. The company designs, manufactures, markets and services a full line of Ford cars, trucks, SUVs, electrified vehicles and Lincoln luxury vehicles, provides financial services through Ford Motor Credit Company and is pursuing leadership positions in electrification, autonomous vehicles and mobility solutions. Ford employs approximately 201,000 people worldwide. For more information regarding Ford, its products and Ford Motor Credit Company, please visit www.corporate.ford.com.

Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 54,000 employees at its wholly owned facilities and approximately 69,000 people when joint ventures and unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 24 manufacturing facilities (16 wholly owned or consolidated joint venture facilities and eight unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911

Contacts:; Toby Sheehan. Ford of Europe. +44 (0) 7384 45473. tsheeha6@ford.com.